## Edge Computing

As outlined in the previous subsection that utilising large and distant Cloud data centres is not beneficial for enhancing mobile nodes’ computational capabilities. To overcome such limitations, the idea of deploying small data centres at the edge of the network was introduced, with an umbrella term called Edge computing. Edge computing is a new form of Cloud computing that brings services close to end users. By moving resources and services within the edge of the network, Edge computing minimises the load on the network and re- duces the response time [35]. Obtaining high bandwidth, ultra-low latency, and real-time access are the main objectives of the Edge computing. The Edge computing characteris- tics include geographical distribution, mobility support, location awareness, proximity and context-awareness [35].

There are different models for the Edge computing in the literature. For example, [36] summarises three types of Edge models. First, Edge Server approaches where servers are deployed to provide computing and storage services to a variety of Edge devices such as sensors and mobile nodes. The servers may refer to real servers machine or Raspberry Pis as long as these devices provide a minimum computing and storage capacity to other devices. Second, Edge Cluster approaches where divers devices are coordinating together to perform different tasks such as smart homes scenario. The third type is the layered approach where Edge devices are layered based on increasingly resources capabilities or location such as smart cities [36]. The Edge computing, in general, has been defined in the literature with different terms. In the following subsections, an overview of such terms is presented with more focus on the term that was considered in this work.

### Cloudlet

An early work, which suggests smartphone users to benefit from the Cloud resources at  the edge of the network, is the Cloudlet [37]. The Cloudlet concept involves nearby (one wireless hop) computing infrastructure that can be utilised by mobile devices. Cloudlets  are decentralized data-centres placed at different places, such as a coffee shop, closer to the mobile users such that compute cycles and storage resources can be leveraged by nearby mobile users. One disadvantage of the Cloudlet is that it is supposed to be accessed via, mostly, by the mobile devices through a WiFi connection. Each time the user wants to access the Cloudlet, it has to switch from the mobile network such as 4G/3G to Wi-Fi whenever the Cloudlet services are exploited [28]. The other disadvantage is the QoS is not guaranteed as the Cloudlet is not part of the mobile network and the mobile devices might not be in  the coverage of the Cloudlet [28]. More specifically, Cloudlet is dependent on robust and uninterrupted Internet connection [34].

### Fog Computing

Another architecture that was proposed for Edge computing is the Fog computing [38]. The Fog computing was proposed in 2012 by Cisco to provide a platform for the Internet of Things (IoT) and big data applications [38]. In Fog computing, task processing is mainly carried out in the local area network and in an IoT gateway or a fog node [34]. The Fog is satiable for applications that require low latency as in gaming application, location awareness as in augmented reality, geo-distributed and large number of nodes as in sensor networks [38]. Fog computing has some disadvantages due to its dependency on wireless connection which has to be live in order to perform any processing and the locality of the intelligence i.e. the intelligence is kept within the local network rather than the mobile network [38] .

### Mobile-Edge Cloud Computing

Quality of Service (QoS) and Quality of Experience (QoE) for mobile nodes can be hardly guaranteed by using the previous mentioned paradigm as the Cloudlet and Fog computing are not integrated into the architecture of the mobile network [28]. Thus, a new edge computing architecture named Mobile Edge Computing or Multi-Access Egde Computing (MEC) to be deployed and integrated with the mobile network was proposed. This terms and its related architecture will be used mainly in this thesis. In the following section, a background about the MEC will be provided.

# Mobile-Edge Cloud Computing

MEC overcomes the limitation of MCC by bringing the Cloud resources (processing and storage resources) to the edge within the Radio Access Network (RAN) [34]. MEC with the concept of integrating Edge computing into the mobile network architecture was devel- oped in 2014 by a Industry Specification Group (ISG) within European Telecommunications Standards Institute (ETSI) [39]. The standardization of the MEC involved mobile opera- tors (e.g., DOCOMO, Vodafone, TELECOM Italia) and manufactures (e.g., IBM, Nokia, Huawei, Intel) [28]. According to ETSI [39], MEC is defined as follows: “Mobile edge computing provides an IT service environment and Cloud computing capabilities at the edge of the mobile network, within the radio access network (RAN) and in close proximity to mobile subscribers.” The main distinguishing features of the MEC, different from the Cloud, are list below [40].

Proximity close to the source of information being easier to capture big data for for analytics tasks. Also, this is beneficial for compute-hungry devices such as augmented reality and video analytics.

Lower Latency As MEC nodes are deployed near to end users, it considerably reduces latency and provides higher bandwidth.

Location Awareness Low-level signaling information for edge devices can be utilised for a local service. This allows for new use cases and new services.

## Architectures of MEC

The general architecture of MEC is shown in Figure 2.2 [34]. As it can be seen, the overall architecture consists of three main layers: mobile nodes, MEC servers and distant Cloud data centres. In the lowest layer, different types of mobile nodes including smartphones, wearable devices and smart vehicles can benefit from the upper layers, i.e. the MEC servers. In the second layer, we have the MEC servers deployed usually within the mobile network in BS or RSU. The mobile nodes can connect to the MEC server utilising the RAN. Moving to the core of the network, we have the large data centres that can be also utilised by the MEC servers in the case of the high volume of the workload. The MEC servers are connecting to the Cloud via fiber link [6, 41]. Also, the MEC servers can be connecting to each other, for example by wired network or via X2 link [41].