Introduction

Mobile-Edge Cloud Computing (MEC) refers to a computing paradigm that brings Cloud’s computing resources closer to the user at the edge of the network.  MEC intends to relocate the Cloud computing resources to the radio access network to optimize the delivery of con-tent and applications to end users.  It involves deploying small data centres (servers) at the edge of the network in locations such as base stations or Road Side Units (RSU). MEC servers can be an advantage for various types of mobile nodes and applications including computation offloading for computation-hungry applications such as in Augmented Reality(AR) applications. Further, MEC servers can be an intermediate data-processing layer for data offloaded by mobile nodes.

MEC servers within the RSUs can play significant roles in improving the performance of mobile vehicular terminals, e.g. Autonomous Vehicles (AV) [5]. The AV can run intelligent vehicle control, traffic management and interactive applications using the built-in computa-tion units.  AVs are equipped with a massive number of sensors that collect contextual data for different types of applications such as transportation systems and navigation applications.  The AVs can collect and sense contextual data and apply different algorithms for data analytics tasks.  An autonomous driving vehicle, for example, produces and consumes ap-proximately 40 terabytes of data per eight driving hours (e.g., a city’s High Definition (HD)map is approximately 1.5TB) [7].  However, despite AVs typically include on-board units, they have small-scale computing and storage resources because of which they are dependent on other computational resources [8].  Also, such applications may require significant computation resources and constrained time delays [6]. The AV then would be required to offload such tasks to one of the MEC servers to enhance its resources capabilities and to meet the applications’ requirements.

Consider MEC servers deployed within RSUs as proposed in [9] with mobile nodes passing by these servers. These servers provide computing resources for task offloaded by mobile nodes. The mobile node can be a smart vehicle or a passenger on board who is running different types of applications. The key problem is the offloading decision by which the mobile node selects an edge server to offload the computing task as the MEC servers’ load have large variation, e.g. sometimes there is a large number of users concurrently using the same server, whereas some other times only a few users are connecting [10].  In other words, the workload of such servers may be different over time [9, 10]. The selection of where (which MEC server) and when(time) to offload has a significant impact on satisfying the requirements of the AV applications [8].  Meanwhile, computing tasks or data gathered by AVs tend to have strict timeliness requirements which may result in the data becoming out-of-date [11]. Further, the existing studies in the area of computation offloading have mainly focus on the decision of whether the computing task should be offloaded or executed locally and a few studies have considered the selection of where a task should be offloaded as in [9, 12] . It is, therefore, vital to define rules by which the mobile node can select a suitable MEC server to be utilized for task offloading. Once an offloading decision has been made and as the mobile node moves, should the offloading happen immediately or would rather delay the offloading for later in order to find a superior MEC server in terms of computing performance?

For example, a naive centralized offloading method can be considered in which the vehicle can request, from a centralized server, the information about the possible MEC servers that can be used for offloading along the road [9, 13].  The centralized server can be located in a higher layer and it connects to all MEC servers with wired network connectivity [9, 13]. However, such architecture is not visible as this might introduce load on the network as the number of vehicles in the road increases [9].  Another solution would be using Vehicle to Vehicle communication (V2V) as discussed in [6], where a vehicle can send tasks to the MEC server (using V2V technology) that it will pass by at the time the computational task finishes. Such solution requires the presence of other vehicles, which is not guaranteed all the time.  Thus, the challenge now is how to optimize the decision of selecting the MEC server if the previous methods are not available and the mobile node only knows about the MEC server it is observing. In other words, the mobile node does not have a global information (or the mobile node has incomplete information) about the candidates MEC servers to be used for offloading.

In this case, we consider two important factors that can be used in order to delay the offloading in the light of finding a better MEC server.  First, we can consider the mobility as an advantage to optimize the decision of which MEC servers to offload.  As the speed of the vehicle increases, the probability of having better MEC server with low workload increases [14].  Second, there is usually a certain deadline for the computational task which gives an opportunity for the decision maker to delay and explore more options for offloading [9, 15].